2022+ Model FastCOLOUR Printer Default Settings

| Setting | F1-1800 1800mm Single Head |  | F1-1800D 1800mm Double Head |
| --- | --- | --- | --- |
| Factory Settings |  |  |  |
| Station | Lift |  |  |
| Wiper | Pop, Lift |  |  |
| Printing Width | 195cm |  | 190cm |
| Raster Type | B |  |  |
| Cart Motor | DC |  | Servo |
| Paper Motor | Step |  |  |
| Wiper Direction | Normal |  |  |
| Paper Direction | Reverse |  |  |
| Car Direction | Cannot Change. If Cart Move Wrong Direction, Swap Wires |  |  |
| Acc Distance | 80mm |  |  |
| Paper Speed | 50 |  |  |
| LCD Type | B |  |  |
| UV Type | D |  |  |
| Auto Clean | Spit: Printhead Flush Ink Only Clean: Full Cleaning Sequence |  |  |
| Wiper Speed | Between 60 to 70 |  |  |
| Station Speed | 84 |  |  |
| Do Wipe Speed | 2 |  |  |
| Fan in Mode | 可控 (Adjustable in Menu) 常开 (Always On) |  |  |
| Head Width | 50mm (for XP600 Printhead |  |  |
| Dry Set | Dry: For UV Printer, Control LED Lamps Heat: For Solvent/Sublimation etc (P.S Still need to adjust wires) |  |  |
| Advance Setup | (Password Protected) |  |  |
| Voltage Adjust | 560 |  |  |
| Voltage Type | S 097f0600 |  |  |
| Colour | KYCcMm |  |  |
| Colour Dual Seq. |  |  | KKCCMMYYCCMM |
| Print Empty | Not Print Empty Pass |  |  |
| Print more | 0 |  |  |
| Spit Switch | Close |  |  |
| Spit Dots | 10 |  |  |
| Spit Empty Data | 3 |  |  |
| Chang config | All Colour |  |  |
| White Repeat |  |  | 1 Pass (Increase Passed for Density) |
| UV Lamp Control | Left LED | Both LEDs (Left & Right) |  |
| R to L On | 1800 |  | 3600 |
| R to L Off | 8000 |  | 8500 |
| L to R On | 3600 |  | 3600 |
| L to R Off | -1800 |  | 0 |
| Distance | 5200 |  | 9500 |