How to Adjust CMA ITV1000/2000/3000 Electro-Pneumatic Regulator

Video is here: https://www.youtube.com/watch?v=ZdLlOVKGWkE

1. 	Engage blow setting to switch on the valve.

2.	⁠Hold unlock button for 5 seconds (far left button). Once unlocked, select S 
 	button (middle button) to set parameters.

- F_1 parameter = minimum output pressure.

- F_2 parameter = maximum output pressure.

3. 	Use arrow keys to set the pressure values for minimum and maximum output.

4. ⁠	once set press the S button (middle button) and disengage Blow function.